Thevenin’s theorem for AC circuit

Thevenin’s theorem for AC circuits states that any linear, two-terminal AC network (containing voltage/current sources, resistors, inductors, and capacitors) can be replaced by an equivalent circuit containing a single sinusoidal voltage source (Vth) in series with a single impedance (Zth). The equivalent impedance is calculated by deactivating independent sources (shorting voltage sources, opening current sources).

Key Steps for AC Thevenin Analysis:

- Remove Load Impedance: Disconnect the load (ZL) from the terminals (A and B).

- Find Vth: Calculate the open-circuit voltage across terminals A and B, using techniques like phasor analysis, nodal analysis, or mesh analysis.

- Find Zth: Determine the equivalent impedance looking back into the terminals with all independent sources deactivated (short-circuit voltage sources, open-circuit current sources).

- Construct Equivalent Circuit: Draw a single voltage source Vth in series with Zth and reconnect the load ZL

Norton's theorem

Norton's theorem for AC circuits states that any linear, two-terminal, bilateral network consisting of sources and impedances can be replaced by an equivalent circuit of a single constant current source IN in parallel with a single impedance ZN. It simplifies complex AC analysis by reducing the network to two components at a specific frequency, facilitating load variations.

Steps to Apply Norton's Theorem in AC Circuits

- Identify Terminals: Remove the load impedance ZL from the terminals (A and B) of the linear circuit.

- Calculate IN (Norton Current): Find the short-circuit current flowing between terminals A and B, typically using mesh or nodal analysis with complex numbers.

- Calculate ZN (Norton Impedance): Determine the equivalent impedance between terminals A and B with all independent sources set to zero (voltage sources shorted, current sources opened).

- Draw Equivalent Circuit: Connect the Norton current source IN in parallel with the Norton impedance ZN and the load ZL Calculate Load Current IL

- Use the current divider rule to find the current through the load:

Find the Norton’s equivalent of the following circuit.